# Cable Automation Script for OrCAD - User Guide

## Overview

TCL script for automated cable drawing in OrCAD Capture from CSV data files.

## Requirements

- OrCAD Capture with TCL support

- TCL_CABLE.OLB library file in the same directory as the script

- A3-sized schematic pages with millimeter units

## CSV File Structure

### Header Section (at file beginning)

text

ProjectNumber,<your_project_number>

NameRightSide,<right_side_name>

NameLeftSide,<left_side_name>

PAGE,<page_name>

### Wire Data Section (after PAGE)

Columns in order (comma-separated):

- SignalName - Signal name

- RightConnector - Right connector (format: "CONNECTOR_NAME/PIN_NAME")

- LeftConnector - Left connector (format: "CONNECTOR_NAME/PIN_NAME")

- InOut - Signal direction: "IN" or "OUT"

- WireType - Cable type:

  - "SINGLE" or "SN" or empty - Single wire

  - "TWISTED" or "TW" - Twisted pair

  - "SHIELDED TWISTED" or "ST" - Shielded twisted pair

- WidthLeftGauge - Wire gauge (left side)

- Color - Wire color (auto-detected if empty)

- WidthRightGauge - Wire gauge (right side)

- WireRightOffset - Right wire offset (number)

- WireInRightGroupe - Right wire group

- WireLeftOffset - Left wire offset (number)

- WireInLeftGroupe - Left wire group

### Format Notes:

- Empty lines separate pages

- Connector format: "CONNECTOR_NAME/PIN_NAME"

- Pin grouping: A1-A9, B1-B9, C1-C9, D1-D9

## Example CSV File

ProjectNumber,CABLE-001

NameRightSide,SOURCE_DEVICE

NameLeftSide,DESTINATION_DEVICE

PAGE,Page1

Signal1,CONN1/A1,CONN2/B1,IN,SINGLE,0.5,RED,0.5,,,,

Signal2,CONN1/A2,CONN2/B2,OUT,SINGLE,0.5,BLUE,0.5,,,,

SPACE,,,,,,,,,,,

Signal3,CONN1/B1,CONN3/C1,IN,TWISTED,0.75,,0.75,1,,1,

Signal4,CONN1/B2,CONN3/C2,OUT,TWISTED,0.75,,0.75,1,,1,

PAGE,Page2

Signal5,CONN4/A1,CONN5/A1,IN,ST,1.0,BLACK,1.0,,,,

## Execution

Script is launched from Python application by setting the global variable path_to_csv_file with the CSV file path.

## Output

The script generates on specified pages:

- Wires between connectors

- Shield and twisted pair notations

- Wire color and gauge labels

- Connector rectangles

- Off-page connectors for input/output signals

## Error Handling

- Detailed logging in script_safe.log file

- Coordinate boundary checking

- Automatic color detection based on signal name patterns

# Cable Automation System - Complete Guide

## Overview

This system automates the creation of cable schematics in OrCAD Capture using Excel data files. It consists of a TCL script for OrCAD and a Python GUI application for data management and validation.

## File Structure

project/
├── scripts/
│ ├── cable.tcl # Main OrCAD automation script
│ └── run_script.tcl # Script runner
├── data/
│ └── cable.csv # Generated cable data file
├── cable_automation_tab.py # Main application GUI
├── cable_validator.py # Data validation module
└── TCL_CABLE.OLB # OrCAD library with symbols

text

## Excel Data File Preparation

### Required Header Section

ProjectNumber,31-667
NumberCable,102
NameLeftSide, SIMULATOR
NameRightSide, SELFTEST
Title,ELEC. SCHEM. - W101 SELFTEST EXTERNAL CABLE
PartNumber,P/N: 000000
DocumentNumber,DOC-000000
Revision,00

### Page Structure

Each cable page starts with:

PAGE,PAGE2

### Wire Data Columns

Signal name,Right connector,Left connector,Right side IN/OUT,Type,"Left wire width, Gauge",Color,"Right wire width, Gauge",Right wire offset,Right groupe,Left wire offset,Left groupe

### Column Definitions

1. **Signal name** - Unique wire identifier (e.g., "SPARE MOTOR P11")

2. **Right connector** - Right-side pin in "CONNECTOR/PIN" format (e.g., "L_P14/A")

3. **Left connector** - Left-side pin in "CONNECTOR/PIN" format (e.g., "L_P14/A")

4. **Right side IN/OUT** - Signal direction: "IN" or "OUT"

5. **Type** - Wire type: "SINGLE", "TWISTED", or "SHIELDED TWISTED"

6. **Left wire width, Gauge** - Wire gauge (e.g., "20")

7. **Color** - Wire color (e.g., "WHITE", "BLACK")

8. **Right wire width, Gauge** - Right side wire gauge

9. **Right wire offset** - Vertical offset rules:

- **Empty** = Straight wire

- **"0"** = Wire with splice component

- **"1", "2", etc.** = Offset upward by specified steps

- **"-1", "-2", etc.** = Offset downward by specified steps

10. **Right groupe** - Right side group identifier

11. **Left wire offset** - Same rules as right offset

12. **Left groupe** - Left side group identifier

## Wire Type Codes

- **SINGLE** or **SN** - Single wire

- **TWISTED** or **TW** - Twisted pair (must be 2 consecutive wires)

- **SHIELDED TWISTED** or **ST** - Shielded twisted pair (must be 2 consecutive wires)

## Automatic Color Detection

If Color column is empty, script automatically assigns:

- **BLACK** for signals containing: "_RXL", "_TXL", "_RXN", "_TXN", "GND", "RTN", "GND_", "_TDN", "_RDN", "_N_", "_L_", "_N", "_L"

- **WHITE** for all other signals

## Data Validation Rules

### Before running the script, the system validates:

1. **Structure Validation**

- Required headers present

- Pages defined correctly

- Wire data exists

2. **Wire Type Rules**

- Twisted pairs must consist of 2 consecutive wires

- Same wire type for both wires in a pair

3. **Connector Pin Rules**

- Pin format must be "CONNECTOR_NAME/PIN_NAME"

- Pins must be unique within each connector type (left/right separate)

- Same pin can exist in both left and right connectors

- No duplicate pins within the same connector

4. **Signal Name Rules**

- Signal names must be unique within each page

- Duplicate names across pages generate warnings

5. **Offset Validation**

- Offsets must be integers

- Valid values: "", "0", "1", "-1", "2", "-2", etc.

6. **Page Capacity**

- Maximum 100 wires per page (configurable)

## Application Usage

### 1. Load Excel File

- Click "Load from Excel" to select your cable data file

- Application loads data into the table view

- Pages are automatically detected and listed in dropdown

### 2. Data Validation

- Click "Run script" to start validation

- System checks all validation rules

- Errors must be fixed before proceeding

- Warnings can be bypassed with confirmation

### 3. Edit Data

- Click "Edit in Excel" to open file in Microsoft Excel

- Make required corrections to data

- Reload file after editing

### 4. Generate Schematic

- After successful validation, script runs automatically

- Creates cable schematic in OrCAD Capture

- Places wires, connectors, splices, and text labels

- Handles offsets and twisted pairs automatically

## OrCAD Requirements

### Page Setup

- Active page must be A3 size (420x297mm)

- Units must be millimeters

- Page must be empty before running script

### Library Requirements

- `TCL_CABLE.OLB` library must be available

- Contains SPLC (splice) and OffPage symbols

## Error Handling

### Common Issues and Solutions

1. **"Page is not A3 size"**

- Ensure active page is A3 (420x297mm)

- Check page units are millimeters

2. **"Objects found on page"**

- Clear the page before running script

- Script requires empty page

3. **"Duplicate pin in connector"**

- Check for repeated pin names in same connector

- Ensure pin names are unique per connector

4. **"Twisted pair must have 2 wires"**

- TW/ST wires must be in consecutive pairs

- Add the second wire or change type to SINGLE

5. **"Invalid offset value"**

- Offsets must be integers or empty

- Remove non-numeric characters from offset columns

## Output Features

### Automatic Placement

- Wires with proper routing and offsets

- Connector rectangles with labels

- Splice components where needed

- Shield symbols for grouped wires

- Color and gauge text labels

- Signal name labels

### Visual Elements

- Left and right connector blocks

- Wire color coding

- Twisted pair indicators

- Shield connections

- Off-page connectors

## Tips for Best Results

1. **Plan Wire Order** - Group related wires together

2. **Use Consistent Naming** - Follow naming conventions for automatic color detection

3. **Check Offsets** - Use offsets to avoid wire crossings

4. **Validate Early** - Run validation frequently during data preparation

5. **Backup OrCAD Files** - Save project before running automation

## Support

For issues with the cable automation system, check:

- Data validation errors in application log

- OrCAD script execution log

- Page setup requirements

- Library file locations